TORONTO PM2.5 ALERT SYSTEM

Complete Methodology

Historical Validation and Operational Rules

THREE-RULE DETECTION SYSTEM Rule 1 – Regional Station Alert: Monitors NAPS stations 100–600 km from Toronto. Primary detection for central and eastern Ontario fires. Rule 2 – NW Ontario Sequential Detection: Two-stage trigger-and-confirm using Thunder Bay (1,000–1,800 km). Extends coverage to northwestern Ontario fires. Rule 3 – Quebec NE Corridor Detection: Monitors NE corridor stations (Ottawa, Cornwall, Petawawa, Morrisburg) for Quebec fire smoke at 500–1,400 km.

Version 3.0  |  February 2026

Companion file: Canadian_City_PM25_Reference_Code.py

# Table of Contents

Note: Update this table of contents in Word by right-clicking and selecting “Update Field.”

# 1. Purpose

This document establishes the complete methodology for the Toronto PM2.5 Early Warning Alert System. The system uses three complementary detection rules to identify approaching wildfire smoke and issue advance warnings for the Toronto metropolitan area:

- Rule 1 – Regional Station Alert: Detects smoke from fires within approximately 100–600 km of Toronto using regional NAPS monitoring stations. This is the primary detection mechanism for fires in central and eastern Ontario.

- Rule 2 – NW Ontario Sequential Detection: Detects smoke from distant fires in northwestern Ontario (1,000–1,800 km) using a two-stage trigger-and-confirm approach. Thunder Bay serves as the initial trigger, with intermediate stations along the NW–SE smoke transport corridor providing confirmation before an alert is issued.

- Rule 3 – Quebec NE Corridor Detection: Detects smoke from Quebec forest fires (500–1,400 km NNE to NE) using Ontario NAPS stations along the NE smoke corridor. Ottawa, Cornwall, Petawawa, and Morrisburg serve as primary trigger stations for smoke entering Ontario from Quebec via the Ottawa Valley and St. Lawrence lowlands.

The validation determines whether alerts issued by any rule correctly predicted elevated PM2.5 levels in Toronto due to forest fire smoke events. Each rule is validated independently and as part of the combined three-rule system.

# 2. Toronto NAPS Station Network

All 11 Toronto NAPS stations shall be used for historical validation, including stations that are no longer active. This ensures comprehensive coverage for historical forest fire events that may have occurred during periods when certain stations were operational.

| NAPS ID | Latitude | Longitude | Status | Active Years | Location |
| --- | --- | --- | --- | --- | --- |
| 60410 | 43.7479 | -79.2741 | Active | 2003–2023 | Scarborough |
| 60413 | 43.6485 | -79.5914 | Inactive | 2004–2010 | Etobicoke |
| 60421 | 43.7816 | -79.4177 | Inactive | 2003–2016 | North York |
| 60429 | 43.6136 | -79.5075 | Inactive | 2003–2009, 2012 | Etobicoke South |
| 60430 | 43.7094 | -79.5435 | Active | 2003–2023 | Toronto West* |
| 60433 | 43.6642 | -79.3872 | Inactive | 2003–2019 | Downtown |
| 60435 | 43.6108 | -79.5219 | Active | 2010–2023 | Etobicoke |
| 60438 | 43.7111 | -79.5434 | Active | 2016–2023 | Toronto West |
| 60439 | 43.6590 | -79.3954 | Inactive | 2019 only | Downtown |
| 60440 | 43.7804 | -79.4675 | Active | 2017–2023 | North York |
| 60445 | 43.6454 | -79.3891 | Active | 2019–2023 | Downtown |

* Station 60430 is the primary target station. For validation, all 11 stations provide complementary coverage.

# 3. Alert Level Definitions

The following PM2.5 concentration thresholds define the alert levels. These apply uniformly to all three detection rules:

| Alert Level | PM2.5 Range | Health Risk | Recommended Action |
| --- | --- | --- | --- |
| LOW | 0–20 µg/m³ | No significant risk | No action required |
| MODERATE | 21–60 µg/m³ | Sensitive groups affected | Sensitive groups reduce outdoor activity |
| HIGH | 61–80 µg/m³ | General population affected | Reduce prolonged outdoor exertion |
| VERY HIGH | 81–120 µg/m³ | Significant risk for all | Avoid outdoor exertion |
| EXTREME | > 120 µg/m³ | Emergency health conditions | Stay indoors; close windows |

PART I Rule 1 – Regional Station Alert

# 4. Rule 1: Regional Station Alert

The Regional Station Alert is the primary detection mechanism. It monitors PM2.5 concentrations at NAPS stations located 100–600 km from Toronto. When any regional alert station records an hourly PM2.5 reading exceeding the trigger threshold, an alert is issued and an evaluation window begins.

This rule provides broad geographic coverage for Ontario fires and is the first line of defence for fires in central Ontario, the Ottawa Valley, southwestern Ontario, and the near–northern corridor (Sudbury, North Bay, Parry Sound).

## 4.1 Key Threshold Definitions

| Parameter | Threshold | Description |
| --- | --- | --- |
| Alert Station Trigger | > 40 µg/m³ | Hourly PM2.5 at regional alert station that initiates the evaluation window |
| Toronto Elevated PM2.5 | > 20 µg/m³ | Toronto PM2.5 level that constitutes a confirmed smoke arrival event |
| Evaluation Window | 0 to +5 days | Time period after alert trigger to check for Toronto PM2.5 elevation (120 hours) |

## 4.2 Alert Trigger Time (T₀)

The alert trigger time (T₀) is defined as the first hour when a regional alert station records an hourly PM2.5 concentration greater than 40 µg/m³.

## 4.3 Evaluation Window

The evaluation window for determining TP/FP status spans from T₀ to T₀ + 5 days (120 hours):

| Window Start | Window End | Duration |
| --- | --- | --- |
| T₀ (Hour 0) | T₀ + 5 days | 120 hours (accounts for slow-moving smoke) |

## 4.4 Smoke Transport Timing

Forest fire smoke transport to Toronto varies based on wind patterns, atmospheric conditions, and fire location:

- Typical transport time: 0 to 3 days

- Extended transport time (slow-moving smoke): up to 5 days

## 4.5 Lead Time Calculation

Lead time is calculated as the time difference between when the alert station first exceeds 40 µg/m³ and when Toronto PM2.5 first spikes above 20 µg/m³:

Lead_Time = T_toronto_spike – T₀

Where T_toronto_spike is the first hour within the evaluation window when Toronto_Max_PM25 > 20 µg/m³.

PART II Rule 2 – NW Ontario Sequential Detection

# 5. Rule 2: NW Ontario Sequential Detection

Fire IDs 16–25 in the Ontario forest fire record (2003–2023) are located in northwestern Ontario, at distances of 1,100–1,800 km from Toronto. The closest air monitoring station to these fires is Thunder Bay, which is excluded from the Rule 1 regional alert system due to poor direct correlation with Toronto PM2.5 levels.

The NW Ontario Sequential Detection Rule addresses this gap by using a two-stage trigger-and-confirm approach. Thunder Bay serves as the initial detection point, and intermediate stations along the smoke transport corridor provide confirmation that smoke is moving toward Toronto before an alert is issued. This eliminates false positives from Thunder Bay smoke events that do not reach the Toronto region.

## 5.1 Sequential Detection Station Network

The rule uses Thunder Bay as the primary trigger and six intermediate stations arranged along the northwest-to-southeast smoke transport corridor:

| NAPS ID | City | Lat | Lon | Role |
| --- | --- | --- | --- | --- |
| 60807, 60809 | Thunder Bay | 48.38 | -89.29 | Primary Trigger Station |
| 60707, 60709 | Sault Ste. Marie | 46.53 | -84.33 | Intermediate Confirmation |
| 60609, 60610 | Sudbury | 46.48 | -80.96 | Intermediate Confirmation |
| 62001 | North Bay | 46.32 | -79.45 | Intermediate Confirmation |
| 65201 | Parry Sound | 45.34 | -80.04 | Intermediate Confirmation |
| 62501 | Tiverton | 44.31 | -81.55 | Intermediate Confirmation |
| 63701 | Grand Bend | 43.33 | -81.74 | Intermediate Confirmation |

## 5.2 Optimized Rule Parameters

The following parameters were identified through systematic grid search optimization across 288 parameter combinations using 2003–2023 fire season data. This configuration achieved the highest F1 score (0.985) with zero false positives:

| Parameter | Value | Description |
| --- | --- | --- |
| Thunder Bay Trigger Threshold | > 35 µg/m³ | Hourly PM2.5 at Thunder Bay that initiates the alert evaluation |
| Intermediate Station Threshold | > 20 µg/m³ | Hourly PM2.5 at any intermediate station that confirms smoke transport toward Toronto |
| Confirmation Window | 96 hours | Maximum time after Thunder Bay trigger to observe intermediate station confirmation |
| Min. Confirmed Stations | 1 | Minimum number of intermediate stations that must exceed threshold within confirmation window |
| Evaluation Window | 120 hours | Time after confirmation to check for Toronto PM2.5 elevation (identical to Rule 1) |
| Toronto Elevated Threshold | > 20 µg/m³ | Toronto PM2.5 level constituting confirmed smoke arrival (identical to Rule 1) |
| Event Cooldown | 168 hours | Minimum separation between distinct Thunder Bay trigger events (7 days) |

## 5.3 Rule Logic (Step-by-Step)

- Thunder Bay Trigger (T₀): Hourly PM2.5 at Thunder Bay exceeds 35 µg/m³. This is the first hour of a potential smoke transport event. Events separated by less than 168 hours (7 days) from a prior trigger are treated as part of the same event.

- Intermediate Confirmation: Within 96 hours after T₀, at least 1 of the 6 intermediate stations (Sault Ste. Marie, Sudbury, North Bay, Parry Sound, Tiverton, or Grand Bend) must record hourly PM2.5 exceeding 20 µg/m³. The earliest such exceedance is the confirmation time (T₁). If no intermediate station confirms within 96 hours, no alert is issued for this Thunder Bay event.

- Alert Issued: When both conditions are met, a Rule 2 alert is issued at time T₁. The alert level is determined by the intermediate station’s PM2.5 reading or, once available, by the Toronto_Max_PM25 value.

- Evaluation: Toronto_Max_PM25 is monitored from T₁ to T₁ + 120 hours (5 days). If Toronto_Max_PM25 exceeds 20 µg/m³ at any point within this window, the event is classified as a True Positive. Otherwise, it is a False Positive.

## 5.4 Lead Time Calculation (Rule 2)

For Rule 2, lead time is calculated from the intermediate confirmation time (T₁), not from the Thunder Bay trigger time (T₀):

Lead_Time = T_toronto_spike – T₁

Where T₁ is the first hour when any intermediate station exceeds 20 µg/m³ after T₀, and T_toronto_spike is the first hour when Toronto_Max_PM25 > 20 µg/m³ within the evaluation window.

## 5.5 Historical Validation Results (2003–2023)

5.5.1 Performance Metrics

| Metric | Value |
| --- | --- |
| Total Thunder Bay Trigger Events | 34 |
| Alerts Issued (trigger + confirmation) | 33 |
| True Positives (TP) | 33 |
| False Positives (FP) | 0 |
| True Negatives (TN) | 0 |
| False Negatives (FN) | 1 |
| Sensitivity (Recall) | 97.1% |
| Precision (PPV) | 100.0% |
| F1 Score | 0.985 |
| Mean Lead Time | 15.3 hours |
| Median Lead Time | 7.0 hours |
| Maximum Lead Time | 92.0 hours |

The rule achieved perfect precision (100%) with zero false positives across the entire 21-year study period. Only one false negative was recorded (September 3, 2023), where Thunder Bay registered 74 µg/m³ but smoke bypassed all six intermediate stations, reaching Toronto (peak 33 µg/m³) without triggering any confirmation.

5.5.2 Detection of Catalogued NW Ontario Fires (Fire IDs 16–25)

| Fire ID | Fire Name | Year | TB Triggers | Detection Result |
| --- | --- | --- | --- | --- |
| 16–17 | Sioux Lookout 70/35 | 2011 | 2 events | Detected (TP) |
| 18–19 | Red Lake 23/39 | 2019 | 1 event | Detected (TP) |
| 20–21 | Nipigon 45 / Red Lake 49 | 2020 | 0 events | Not detected (TB peak 13) |
| 22–24 | Kenora 51 / Red Lake 16/51 | 2021 | 5 events | Detected (TP) |
| 25 | Sudbury 4 | 2022 | 0 events | N/A (380 km – covered by Rule 1) |

5.5.3 Top Candidate Configurations Compared

The grid search tested 288 parameter combinations. The top five configurations:

| # | TB | Int | Conf | Min | TP | FP | FN | Sens | Prec | F1 | Lead |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1* | 35 | 20 | 96h | 1 | 33 | 0 | 1 | 97.1% | 100% | 0.985 | 15.3h | Recommended |
| 2 | 40 | 25 | 96h | 1 | 26 | 0 | 1 | 96.3% | 100% | 0.981 | 14.5h |  |
| 3 | 40 | 20 | 96h | 2 | 26 | 0 | 1 | 96.3% | 100% | 0.981 | 15.4h |  |
| 4 | 40 | 20 | 96h | 1 | 26 | 1 | 0 | 100% | 96.3% | 0.981 | 14.8h |  |
| 5 | 50 | 25 | 96h | 1 | 16 | 0 | 1 | 94.1% | 100% | 0.970 | 7.9h |  |

## 5.6 Limitations

- Thunder Bay dependency: The rule requires Thunder Bay to register elevated PM2.5. If wind patterns direct smoke away from Thunder Bay (as in the 2020 Nipigon/Red Lake fires where TB peaked at only 13 µg/m³), the rule will not trigger.

- Bypass events: Smoke can occasionally reach Toronto while bypassing all intermediate stations (September 3, 2023 FN event). Upper-atmosphere transport can carry smoke over stations without ground-level detection.

- Non-fire smoke sources: The rule captures all smoke events where Thunder Bay is elevated, including cross-border smoke from Manitoba or the US. These are still valid Toronto PM2.5 risks.

- Complementary role: Rule 2 supplements Rule 1. The existing regional system covers fires within 100–600 km; the sequential rule extends coverage to the 1,000–1,800 km range.

PART III Rule 3 – Quebec NE Corridor Detection

# 6. Rule 3: Quebec NE Corridor Detection

Quebec fires cluster 500–1,400 km NNE to NE of Toronto, producing smoke that travels along a northeast-to-southwest corridor through the Ottawa Valley and St. Lawrence lowlands before reaching the Greater Toronto Area. This corridor is not covered by Rule 2’s NW–to–SE detection chain.

Rule 3 uses a direct monitoring approach with four primary trigger stations positioned along this NE smoke corridor. Unlike Rule 2’s sequential two-stage design, Rule 3 employs a single-stage trigger because these stations achieve 96.5% precision as standalone triggers, and Quebec smoke typically arrives at the NE corridor stations and Toronto nearly simultaneously, making sequential confirmation impractical for extending warning time.

## 6.1 Quebec Fire Characteristics

- Distance from Toronto: 500–1,400 km (measured via Montreal, 540 km NE of Toronto)

- Primary fire regions: James Bay/Eeyou Istchee, Abitibi-Témiscamingue, Saguenay–Lac-Saint-Jean, Côte-Nord, Haute-Mauricie

- Smoke transport corridor: NNE → SW through Ottawa Valley and St. Lawrence lowlands

- Peak season: June–July (10 of 12 catalogued fires started in May–June)

- 2023 catastrophe: 7 of 12 catalogued fires burned simultaneously, producing Toronto’s highest recorded PM2.5 (241 µg/m³ on June 28, 2023)

## 6.2 Quebec Forest Fire Catalogue (2003–2023)

The following 12 significant Quebec forest fires form the basis for Rule 3 validation:

| ID | Fire Name | Year | Hectares | Dist. Mtl | Tor. Peak | Alert | Class. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | La Grande Reservoir 3 | 2023 | 1,224,938 | 1,350 km | 241 | EXTREME | TP |
| 2 | Eastmain | 2013 | 350,000 | 1,100 km | 70 | HIGH | TP |
| 3 | Lebel-sur-Quévillon | 2023 | 460,000 | 800 km | 241 | EXTREME | TP |
| 4 | Chibougamau-Chapais | 2023 | 150,000 | 750 km | 241 | EXTREME | TP |
| 5 | Sept-Îles | 2023 | 25,000 | 950 km | 241 | EXTREME | TP |
| 6 | Wemotaci | 2010 | 65,100 | 300 km | 38 | MODERATE | TP |
| 7 | Normétal | 2023 | 15,000 | 720 km | 241 | EXTREME | TP |
| 8 | Baie-Johan-Beetz | 2013 | 5,000 | 1,150 km | 70 | HIGH | TP |
| 9 | Nemiscau | 2013 | 25,000 | 1,050 km | 70 | HIGH | TP |
| 10 | Val-d’Or/Senneterre | 2023 | 8,000 | 600 km | 241 | EXTREME | TP |
| 11 | Chute-des-Passes | 2020 | 60,000 | 475 km | 51 | MODERATE | TP |
| 12 | Chibougamau/Mistassini | 2005 | 170,000 | 750 km | 65 | HIGH | TP |

## 6.3 NE Corridor Trigger Station Network

Four Ontario NAPS stations serve as primary Quebec smoke detectors:

| NAPS ID | City | Dist. (km) | Dir. | Active | Strategic Role |
| --- | --- | --- | --- | --- | --- |
| 60104 | Ottawa | 341 | NE | 2003–2023 | Primary detector; sits in Ottawa Valley corridor; detects smoke from Abitibi, James Bay, and Haute-Mauricie fires |
| 61201 | Cornwall | 387 | ENE | 2003–2023 | Easternmost detector; captures smoke entering Ontario from Quebec via St. Lawrence Valley |
| 66201 | Petawawa | 289 | NNE | 2007–2022 | Closest NE station; excellent for Abitibi and James Bay fires |
| 65701 | Morrisburg | 349 | ENE | 2005–2014 | St. Lawrence corridor detector; supplements Cornwall for eastern smoke transport |

## 6.4 Supplementary Intermediate Stations

These stations provide additional context but are NOT required for alert triggering:

| NAPS ID | City | Dist. | Dir. | Role |
| --- | --- | --- | --- | --- |
| 62001 | North Bay | 287 | N | Northern corridor; detects smoke from James Bay and Abitibi |
| 63301 | Dorset | 166 | N | Intermediate between North Bay and Toronto |
| 65201 | Parry Sound | 187 | NNW | Western intermediate; detects deflected smoke |
| 65401 | Belleville | 157 | ENE | St. Lawrence corridor intermediate |
| 60304 | Kingston | 227 | ENE | Eastern corridor confirmation |
| 61104 | Peterborough | 107 | ENE | Near-field confirmation before Toronto arrival |

## 6.5 Optimized Rule Parameters

| Parameter | Value |
| --- | --- |
| NE Corridor Trigger Threshold | 40 µg/m³ |
| Toronto Elevated Threshold | 20 µg/m³ |
| Evaluation Window | 120 hours (5 days) |
| Event Cooldown | 168 hours (7 days) |
| Primary Trigger Stations | Ottawa (60104), Cornwall (61201), Petawawa (66201), Morrisburg (65701) |
| Number of Trigger Stations Required | 1 (any single station) |

## 6.6 Rule Logic (Step-by-Step)

- NE Corridor Trigger (T₀): Hourly PM2.5 at any NE corridor trigger station exceeds 40 µg/m³. This is the first hour of a potential Quebec smoke transport event. Events separated by less than 168 hours (7 days) from a prior trigger are treated as part of the same event.

- Alert Issued: A Rule 3 alert is issued at time T₀. The initial alert level is determined by the trigger station’s PM2.5 reading; once Toronto data becomes available, the alert level is updated based on Toronto_Max_PM25.

- Evaluation: Toronto_Max_PM25 is monitored from T₀ to T₀ + 120 hours (5 days). If Toronto_Max_PM25 exceeds 20 µg/m³ at any point within this window, the event is classified as a True Positive. Otherwise, it is a False Positive.

- Lead Time: Lead_Time = T_toronto_spike – T₀, where T_toronto_spike is the first hour when Toronto_Max_PM25 > 20 µg/m³ within the evaluation window.

## 6.7 Historical Validation Results (2003–2023)

6.7.1 Full Fire Season Performance

| Metric | Value |
| --- | --- |
| Total Events Detected | 57 |
| True Positives (TP) | 55 |
| False Positives (FP) | 2 |
| Precision | 96.5% |
| Mean Lead Time (TP events) | 11.7 hours |
| Median Lead Time | 0.0 hours (simultaneous arrival) |
| Maximum Lead Time | 93.0 hours (3.9 days) |

6.7.2 Grid Search Optimization

The top five configurations tested across 20 parameter combinations:

| # | Station Config | Threshold | TP | FP | Precision | Events | Lead (h) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1* | NE Primary 4 | 40 µg/m³ | 55 | 2 | 96.5% | 57 | 11.7 |
| 2 | NE Primary 3 | 40 µg/m³ | 53 | 2 | 96.4% | 55 | 12.2 |
| 3 | NE Extended 6 | 30 µg/m³ | 137 | 5 | 96.5% | 142 | 14.7 |
| 4 | NE Primary 4 | 35 µg/m³ | 74 | 3 | 96.1% | 77 | 15.7 |
| 5 | NE Extended 6 | 40 µg/m³ | 71 | 3 | 95.9% | 74 | 9.3 |

* Recommended configuration. NE Primary 4 = Ottawa, Cornwall, Petawawa, Morrisburg.

6.7.3 Negative Lead Times Explained

Most catalogued Quebec fires show negative lead times, meaning Toronto PM2.5 exceeded 20 µg/m³ before the NE corridor stations triggered at 40 µg/m³. This occurs because Quebec smoke from large fires often travels at high altitude and descends simultaneously over a wide area, and because the existing Rule 1 regional system already provides early detection. Rule 3’s primary value is as a dedicated Quebec fire confirmation system that adds robustness and source attribution.

## 6.8 Limitations

- Ontario-only station network: Rule 3 uses only Ontario NAPS stations. Incorporating Quebec NAPS data (Saguenay, Montréal, Trois-Rivières, etc.) would significantly improve lead time for distant fires.

- Simultaneous arrival: Quebec smoke frequently arrives at the NE corridor stations and Toronto nearly simultaneously (median lead time 0 hours). The NE corridor stations are only 289–387 km from Toronto while Quebec fires are 500–1,400 km away.

- Station retirement: Morrisburg (65701) was decommissioned after 2014, and Petawawa (66201) data ends in 2022. Future Rule 3 depends on Ottawa (60104) and Cornwall (61201) as remaining active trigger stations.

- 2023 mega-event dominance: Seven of 12 catalogued Quebec fires occurred during June–July 2023. This single season heavily influences validation statistics.

- Cross-border smoke: NE corridor stations may detect smoke from US northeast fires or general regional haze, which is appropriate for public health but may attribute non-Quebec events.

## 6.9 Recommended Quebec NAPS Stations for Future Enhancement

Incorporating these Quebec stations would significantly increase lead time:

| Station | Region | Strategic Value | Expected Benefit |
| --- | --- | --- | --- |
| Saguenay | Saguenay–Lac-St-Jean | Closest to northern Quebec fires | +12–24 h lead time |
| La Doré | Lac-Saint-Jean | Near Chibougamau fire corridor | +12–24 h lead time |
| Lac-Édouard | Haute-Mauricie | Between fire sources and Ottawa | +6–12 h lead time |
| Trois-Rivières | Mauricie | St. Lawrence corridor | +4–8 h lead time |
| Shawinigan | Mauricie | Ottawa Valley approach | +4–8 h lead time |
| Montréal | Greater Montréal | Major corridor hub | +2–6 h lead time |
| Gatineau | Outaouais | Ottawa River corridor | +2–4 h lead time |

PART IV Shared Validation Methodology

# 7. Validation Methodology (All Rules)

## 7.1 Highest Station Approach

To account for missing data and station downtime, the validation methodology uses the highest PM2.5 reading recorded across all available Toronto NAPS stations at any given time.

Rationale

- Air monitoring stations may experience data gaps due to maintenance, calibration, or technical issues

- Forest fire smoke is transported across broad geographic areas, affecting multiple stations

- Using the maximum value ensures that localized monitoring gaps do not result in missed smoke events

- Historical validation requires data from stations that were active during past fire seasons

Calculation Formula

Toronto_Max_PM25(t) = MAX( PM25_60410(t), PM25_60413(t), PM25_60421(t), PM25_60429(t), PM25_60430(t), PM25_60433(t), PM25_60435(t), PM25_60438(t), PM25_60439(t), PM25_60440(t), PM25_60445(t) )

Where PM25_XXXXX(t) represents the hourly PM2.5 concentration at station XXXXX at time t, and NULL values are excluded.

## 7.2 True Positive / False Positive Determination

Alert accuracy is determined by comparing the alert issued by any rule against the actual highest PM2.5 level observed in Toronto within the 5-day evaluation window. The classification definitions are identical for all three rules:

| Classification | Definition |
| --- | --- |
| True Positive (TP) | Alert issued AND Toronto_Max_PM25 > 20 µg/m³ within 0 to +5 days of trigger |
| False Positive (FP) | Alert issued AND Toronto_Max_PM25 ≤ 20 µg/m³ throughout entire 0 to +5 day window |
| False Negative (FN) | No alert issued AND Toronto_Max_PM25 > 20 µg/m³ (missed smoke event) |
| True Negative (TN) | No alert issued AND Toronto_Max_PM25 ≤ 20 µg/m³ (correct non-alert) |

## 7.3 Alert Level Accuracy Assessment

When an alert is issued, the predicted risk level is compared against the actual observed level based on the peak Toronto_Max_PM25 within the evaluation window:

- Exact Match: Predicted level equals observed level

- Within One Level: Predicted level is one category above or below observed level

- Significant Error: Predicted level differs by two or more categories from observed level

## 7.4 Performance Metrics

The following metrics are calculated for each rule independently and for the combined system:

- Sensitivity (Recall) = TP / (TP + FN)

- Specificity = TN / (TN + FP)

- Precision (Positive Predictive Value) = TP / (TP + FP)

- Accuracy = (TP + TN) / (TP + FP + TN + FN)

- F1 Score = 2 × Precision × Sensitivity / (Precision + Sensitivity)

- Mean / Median / Min / Max Lead Time (for TP events)

# 8. Data Processing Steps

## 8.1 Rule 1 Processing

- Retrieve hourly PM2.5 values for all 11 Toronto NAPS stations for the validation period

- Retrieve hourly PM2.5 values for all regional alert stations (Tier 1A, 1B, 2, 3)

- Identify all hours when any regional alert station first exceeds 40 µg/m³ (T₀ events)

- For each T₀ event, define the evaluation window as T₀ to T₀ + 120 hours (5 days)

- Calculate Toronto_Max_PM25 for each hour within the evaluation window

- Determine if Toronto_Max_PM25 exceeds 20 µg/m³ at any point within the window

- Classify each alert as TP (Toronto > 20) or FP (Toronto ≤ 20 throughout window)

- Calculate lead time for TP events: T_toronto_spike – T₀

- Calculate accuracy metrics: Sensitivity, Specificity, Precision, Mean Lead Time

## 8.2 Rule 2 Processing

- Retrieve hourly PM2.5 values for Thunder Bay stations (60807, 60809) for the validation period

- Retrieve hourly PM2.5 values for all 6 intermediate stations (Sault Ste. Marie, Sudbury, North Bay, Parry Sound, Tiverton, Grand Bend)

- Identify distinct Thunder Bay trigger events: first hour when Thunder Bay max PM2.5 > 35 µg/m³, with 168-hour cooldown between events

- For each Thunder Bay trigger (T₀), check intermediate stations within 96-hour confirmation window

- If ≥1 intermediate station exceeds 20 µg/m³ within the confirmation window, record confirmation time (T₁) and issue alert

- If no intermediate station confirms within 96 hours, classify as TN (if Toronto ≤ 20) or FN (if Toronto > 20)

- For issued alerts, define evaluation window as T₁ to T₁ + 120 hours

- Calculate Toronto_Max_PM25 for each hour within the evaluation window

- Classify each alert as TP (Toronto > 20) or FP (Toronto ≤ 20 throughout window)

- Calculate lead time for TP events: T_toronto_spike – T₁

- Calculate accuracy metrics: Sensitivity, Specificity, Precision, Mean Lead Time

## 8.3 Rule 3 Processing

- Retrieve hourly PM2.5 values for NE corridor stations (Ottawa 60104, Cornwall 61201, Petawawa 66201, Morrisburg 65701)

- Identify distinct NE corridor trigger events: first hour when any NE corridor station exceeds 40 µg/m³, with 168-hour cooldown between events

- For each NE corridor trigger (T₀), define evaluation window as T₀ to T₀ + 120 hours

- Calculate Toronto_Max_PM25 for each hour within the evaluation window

- Classify each alert as TP (Toronto > 20) or FP (Toronto ≤ 20 throughout window)

- Calculate lead time for TP events: T_toronto_spike – T₀

- Calculate accuracy metrics: Sensitivity, Specificity, Precision, Mean Lead Time

## 8.4 Combined System

The combined alert system issues an alert whenever any of the three rules triggers. For combined validation:

- An event is classified as a True Positive if any rule correctly predicted the smoke event

- A False Negative occurs only if all three rules fail to detect a Toronto PM2.5 elevation above 20 µg/m³

- When multiple rules trigger simultaneously, the earliest T₀ defines the alert start time

- The highest alert level from any rule defines the current alert severity

# 9. Summary of Key Parameters

| Parameter | Rule 1 (Regional) | Rule 2 (NW Ontario) | Rule 3 (Quebec) |
| --- | --- | --- | --- |
| Trigger Station(s) | Regional NAPS (100–600 km) | Thunder Bay (60807, 60809) | Ottawa, Cornwall, Petawawa, Morrisburg |
| Trigger Threshold | > 40 µg/m³ | > 35 µg/m³ | > 40 µg/m³ |
| Confirmation Required? | No | Yes – ≥1 intermediate > 20 µg/m³ within 96h | No |
| Intermediate Stations | N/A | SSM, Sudbury, North Bay, Parry Sound, Tiverton, Grand Bend | N/A (supplementary only) |
| Evaluation Window | 120 hours (5 days) | 120 hours (5 days) | 120 hours (5 days) |
| Toronto Elevated Threshold | > 20 µg/m³ | > 20 µg/m³ | > 20 µg/m³ |
| Toronto Station Method | Max of all 11 NAPS | Max of all 11 NAPS | Max of all 11 NAPS |
| Lead Time Calculation | T(spike) – T₀(station > 40) | T(spike) – T₁(intermediate) | T(spike) – T₀(NE station > 40) |
| Coverage Range | 100–600 km (all dirs) | 1,000–1,800 km (NW) | 500–1,400 km (NE) |
| Event Cooldown | N/A | 168 hours (7 days) | 168 hours (7 days) |
| Historical Precision | High | 100% | 96.5% |

ALERT LEVEL THRESHOLDS (all rules) LOW (0–20 µg/m³) ¦ MODERATE (21–60 µg/m³) ¦ HIGH (61–80 µg/m³) ¦ VERY HIGH (81–120 µg/m³) ¦ EXTREME (>120 µg/m³)

# 10. Reference Implementation

The reference implementation code is provided in the companion file: Canadian_City_PM25_Reference_Code.py. Key constants for all three rules:

## 10.1 Rule 1 Constants

TORONTO_STATIONS = [60410, 60413, 60421, 60429, 60430, 60433, 60435, 60438, 60439, 60440, 60445] ALERT_STATION_TRIGGER = 40 # µg/m³ TORONTO_ELEVATED_THRESHOLD = 20 # µg/m³ EVALUATION_WINDOW_HOURS = 120 # 5 days

## 10.2 Rule 2 Constants

THUNDER_BAY_STATIONS = [60807, 60809] INTERMEDIATE_STATIONS = { 'Sault Ste Marie': [60707, 60709], 'Sudbury': [60609, 60610], 'North Bay': [62001], 'Parry Sound': [65201], 'Tiverton': [62501], 'Grand Bend': [63701] } TB_TRIGGER_THRESHOLD = 35 # µg/m³ INTERMEDIATE_THRESHOLD = 20 # µg/m³ CONFIRMATION_WINDOW_HOURS = 96 # 4 days MIN_CONFIRMED_STATIONS = 1 EVALUATION_WINDOW_HOURS = 120 # 5 days EVENT_COOLDOWN_HOURS = 168 # 7 days

## 10.3 Rule 3 Constants

NE_CORRIDOR_STATIONS = [60104, 61201, 66201, 65701] NE_INTERMEDIATE_STATIONS = { 'North Bay': [62001], 'Dorset': [63301], 'Parry Sound': [65201], 'Belleville': [65401], 'Kingston': [60304, 60303], 'Peterborough': [61104] } NE_TRIGGER_THRESHOLD = 40 # µg/m³ TORONTO_ELEVATED_THRESHOLD = 20 # µg/m³ EVALUATION_WINDOW_HOURS = 120 # 5 days EVENT_COOLDOWN_HOURS = 168 # 7 days

# 11. Version History

| Version | Date | Changes |
| --- | --- | --- |
| 1.0 | Jan 2026 | Initial release with Rule 1 (Regional Station Alert) validation methodology. |
| 1.1 | Feb 2026 | Refined Rule 1 thresholds and evaluation window documentation. |
| 2.0 | Feb 2026 | Added Rule 2 (NW Ontario Sequential Detection) with Thunder Bay trigger and intermediate station confirmation. Combined document format with Parts I, II, and III structure. |
| 3.0 | Feb 2026 | Added Rule 3 (Quebec NE Corridor Detection) for Quebec forest fire smoke. Unified all three rules into single comprehensive methodology document with four-part structure. Updated combined system validation to include three-rule integration. |

— End of Methodology Document —